Lab Report

Embedded Systems

Himanshu Rai

111601032                                              6th September, 2019

Objective

To light up RGB lights on FRDM-KL25Z board, with LED colour determined by user input.

Theory

UART Communication (Universal asynchronous receiver/transmitter)

In UART communication, two UARTs communicate directly with each other. The transmitting UART converts parallel data from a controlling device like a CPU into serial form, transmits it in serial to the receiving UART, which then converts the serial data back into parallel data for the receiving device. Only two wires are needed to transmit data between two UARTs.

We will use UART0 ports on the FRDM board to read user input based on which we can decide the colour with which we can light the LED. The following steps are required to configure the UART0 port so that we can use it to read the inputs.

- By default, UART modules are disabled to save power. So, we should first enable the clocks to the ports corresponding to UART0 to prevent hardware fault. The system clocking gate control register 4 (SIM_SCGC4) gates the clocks to UART ports. We can enable the clock for UART0 by setting the 10th bit in SIM_SCGC4 control register.

- Next we need to choose from the different available clock sources. SOPT2 contains the controls for selecting many of the module clock source options on this device. The clock for UART0 can be configured by modifying bit positions 26th and 27th.

  - 00 - Clock disabled

  - 01 - MCGFLLCLK clock or MCGPLLCLK/2 clock

  - 10 - OSCERCLK clock

  - 11 - MCGIRCLK clock

We have to choose the second option, so we will configure the bits to have 01 at the mentioned bit positions.

- Next we need to set up the SBR and OSR (over sampling ratio). These two are related by the following equation

SBR = clock frequence / (OSR * baud rate)

For us, baud rate = 57600Hz, clock frequency = 22MHz, and we will have OSR of 16. So, SBR will be 24.

We can set OSR through UART0_C4 register using bits 0 to 4. For setting SBR we need to modify two registers - UART0_BDL which corresponds to SBR[7:0] and UART0_BDH whose first 5 bits corresponds to SBR[12:8] .

- Next we will activate port A by setting 9th bit in SIM_SCGC5 register and then configure port A1 to be used as UART_Tx by putting 010 at bit positions 10 to 8 in PCR for port A1.

- Finally activate the receiver of UART0 by setting UART0_C2 register as 4.

We can check whether we have received some input by checking 5th bit in UART0_S1 register. If this is set it means the receive buffer is full and we can see the input in UART0_D register.

Lighting up the LEDs

The following tables give the correspondence between the LEDs and port pins associated with them

| LED | GPIO PIN |
| --- | --- |
| Red | PortB18 |
| Green | PortB19 |
| Blue | PortD1 |

First we will have to follow the following procedure to configure the ports/pins corresponding to the LEDs, before using them

- By default, GPIO modules are disabled to save power. So, we should first enable the clocks to the ports corresponding to the LEDs to prevent hardware fault. The system clocking gate control register 5 (SIM_SCGC5) gates the clocks to GPIO ports. We can enable the clock for these ports by setting the respective bits in SIM_SCGC5 control registers - which are bit 10 (for port B - red and green LEDs) and bit 12 (for port D - blue LED).

- Next configure the pins for GPIO . For this we must write 001 in the corresponding program control registers (PCR) at bit positions 10 to 8.

- Now set the pins as output pins by setting the corresponding pin position in the port data direction register (PDDR) of the corresponding ports.

After setting up the pins, we can turn the LEDs on and off by setting the corresponding bits in port clear output register (PCOR) and port toggle output register (PTOR).

Procedure

According to the steps mentioned above, it would suffice to write a function for activating UART0 port and three other functions for each LEDs - one to initialise the ports and the other two to switch the LEDs on and off.

The initialisation functions for each LEDs and UART0 need to be called only once, at the beginning of the program. Also it is safe to call off function for each LED at the beginning.

Next in a loop we wait for user input and then light up the LED depending on it. We keep some feasible delay and then turn off the LED and then again wait for next input.

The source code is at the end of the report.

Results

The LEDs worked as desired depending upon the user input.

Source Code

#include<MKL25Z4.h>

void UART0_init() {

// enabling UART0 by setting the corresponding bit in SCGC4 register

SIM->SCGC4 |= 1 << 10;

// enable the clock at UART0 from FLL or PLL/2 by putting 01 at bit

// positions 27 and 26 in SOPT2 registers

SIM->SOPT2 |= 1 << 26;

SIM->SOPT2 &= ~(1 << 27);

/*

SBR = (clock frequency = 22000000) / (OSR * (baud rate = 57600))

clock frequency and baud rate are fixed so we need only OSR or SBR

SBR is of 13 bits and we set its value using the following registers -

SBR[7:0] = BDL[7:0] and SBR[12:8] = BDH[4:0]

*/

// disable all the operation modes in UART0

UART0->C2 = 0x00;

// SBR setting for baud rate of 57600 (and OSR = 16)

UART0->BDH = 0x00;

UART0->BDL = 0x18;

UART0->C4 = 0x0F;       // for an OSR of 16

UART0->C1 = 0x00;       // no parity

// activate the receiver of UART0

UART0->C2 = 0x04;

// activating port A

SIM->SCGC5 |= 1 << 9;

// configuring PTA1 as UART_Tx by putting 010 at bit positions 10 to 8 in PCR for PORT A1

/*

Note that we can also do this using alternative 4 for PORTE21

But here we are using alternative 1 for PORTA1 because it is connected to USB port

whereas PORTE20 is not

*/

PORTA->PCR[1] &= ~(7 << 8);

PORTA->PCR[1] |= 1 << 9;

}

void delay(int n) {

while(n--);

}

void led_red_init() {

SIM->SCGC5 |= 1 << 10;        // will activate the port B

// set the pin as gpio

PORTB->PCR[18] &= 0xFFFFF8FF;   // set 8th, 9th and 10th bit of PCR[18] to 0

PORTB->PCR[18] |= 1 << 8;       // set 8th bit of PCR[18] to 1

// set the port as output port

PTB->PDDR |= 1 << 18;           // set the 18th bit of PDDR to 1 for output

}

void led_red_on() {

// clear the 18th bit of PDOR register

PTB->PCOR |= 1 << 18;           // set the 18th bit of PCOR to 1

}

void led_red_off() {

// toggle the 18th bit of PDOR register

PTB->PTOR |= 1 << 18;

}

void led_green_init() {

SIM->SCGC5 |= 1 << 10;        // will activate the port B

// set the pin as gpio

PORTB->PCR[19] &= 0xFFFFF8FF;   // set 8th, 9th and 10th bit of PCR[19] to 0

PORTB->PCR[19] |= 1 << 8;       // set 8th bit of PCR[19] to 1

// set the port as output port

PTB->PDDR |= 1 << 19;           // set the 19th bit of PDDR to 1 for output

}

void led_green_on() {

// clear the 18th bit of PDOR register

PTB->PCOR |= 1 << 19;           // set the 19th bit of PCOR to 1

}

void led_green_off() {

// toggle the 18th bit of PDOR register

PTB->PTOR |= 1 << 19;

}

void led_blue_init() {

SIM->SCGC5 |= 1 << 12;        // will activate the port D

// set the pin as gpio

PORTD->PCR[1] &= 0xFFFFF8FF;    // set 8th, 9th and 10th bit of PCR[1] to 0

PORTD->PCR[1] |= 1 << 8;        // set 8th bit of PCR[1] to 1

// set the port as output port

PTD->PDDR |= 1 << 1;            // set the 1th bit of PDDR to 1 for output

}

void led_blue_on() {

// clear the 18th bit of PDOR register

PTD->PCOR |= 1 << 1;            // set the 1th bit of PCOR to 1

}

void led_blue_off() {

// toggle the 18th bit of PDOR register

PTD->PTOR |= 1 << 1;

}

int main() {

SystemCoreClockUpdate();    // updating the clock from PLL

UART0_init();

led_red_init();

led_green_init();

led_blue_init();

led_red_off();

led_green_off();

led_blue_off();

while(1) {

while(!(UART0->S1 & (1 << 5))); // wait while transmit data register is empty

char ch = UART0->D;

if(ch == 'A') led_red_on(), delay(1000000), led_red_off();

else if(ch == 'B') led_green_on(), delay(1000000), led_green_off();

else led_blue_on(), delay(1000000), led_blue_off();

}

}